PhysicsNeMo Aneurysm PoC — Consolidated Notes

Exhaust Pipe Thermal Extension  |  June 2026

# 1. Modified Parameters (Custom Geometry)

The following parameters were changed from the original NVIDIA aneurysm example to match the custom geometry. Two values (inlet_normal, outlet_area_raw) still require verification from the STL/CAD files.

| Parameter | Original Value | New Hardcoded Value | Location in Code |
| --- | --- | --- | --- |
| nu | 0.025 | 7.989e-05 | nu = 7.989e-05 |
| inlet_vel | 1.5 | 1.5 (unchanged) | inlet_vel = 1.5 |
| center | (-18.40, -50.28, 12.84) | (0.01151354, 0.03322, 0.0) | center = (0.01151354, 0.03322, 0.0) |
| scale | 0.4 | 12.753 | scale = 12.753 |
| inlet_center | (-4.24, 4.08, -4.63) | (-0.1469, -0.4996, 0.0) | inlet_center = (-0.1469, -0.4996, 0.0) |
| inlet_area_raw | 21.1284 | 5.0179867253e-05 | inlet_area_raw = 5.0179867253e-05 |
| inlet_area | 21.1284 x 0.4^2 | inlet_area_raw * scale^2 | inlet_area = inlet_area_raw * (scale**2) |
| inlet_radius | sqrt(21.1284/pi) x scale | 0.003994 (raw) / 0.05094 (scaled) | inlet_radius = np.sqrt(inlet_area / np.pi) |
| inlet_normal | (0.8526, -0.428, 0.299) | Verify from STL | inlet_normal = (...) |
| outlet_area_raw | 12.0773 | Provide actual value | outlet_area_raw = ... |
| Q_scaled | 2.540 | 0.006122 | Q_scaled = 0.006122 |
| Re | Not defined | 150 | Achieved via nu = 7.989e-05 |

# 2. Temperature Extension — PoC Setup

The solver has been extended with an advection-diffusion equation for temperature to simulate exhaust pipe thermal flow using the aneurysm geometry as a stand-in.

## 2.1 Thermal Parameters

| Parameter | Value | Notes |
| --- | --- | --- |
| Pr | 0.7 | Exhaust gas / air-like |
| D (thermal diffusivity) | nu / Pr ~ 1.141e-04 | Scales with nu / Pr |
| T_inlet (physical) | 600 degC | Hot exhaust gas at inlet |
| T_wall (physical) | 100 degC | Cooled pipe wall |
| T_inlet_nd (PINN) | 1.0 | Non-dimensional inlet |
| T_wall_nd (PINN) | 0.0 | Non-dimensional wall |

## 2.2 Physical-to-Non-Dimensional Mapping

The PINN solves for a normalized temperature T_nd in [0, 1]. To recover physical temperatures after solving:

T_physical  =  T_wall  +  T_nd x (T_inlet - T_wall)

=  100  +  T_nd x 500   [degC]

## 2.3 Boundary Condition Choices

Wall BC: Fixed temperature (Dirichlet) — more stable for PINN training than Neumann (zero-flux). Physically appropriate as surrounding material acts as a thermal reservoir.

Outlet BC: Temperature is left free (no T constraint) — advection carries it naturally out of the domain. Only pressure (p=0) is enforced at the outlet.

# 3. Complete Code Changes

Add the following import at the top of the file:

from physicsnemo.sym.eq.pdes.advection_diffusion import AdvectionDiffusion

Then apply these changes to the run() function:

# Fluid properties nu = 7.989e-05 Pr = 0.7 D = nu / Pr # ~1.141e-04 T_inlet_nd = 1.0 # non-dim inlet temperature T_wall_nd = 0.0 # non-dim wall temperature # PDE nodes ns = NavierStokes(nu=nu * scale, rho=1.0, dim=3, time=False) ad = AdvectionDiffusion(T='T', rho=1.0, D=D * scale, dim=3, time=False) # Network (add T to outputs) flow_net = instantiate_arch( input_keys=[Key('x'), Key('y'), Key('z')], output_keys=[Key('u'), Key('v'), Key('w'), Key('p'), Key('T')], cfg=cfg.arch.fully_connected, ) nodes = ( ns.make_nodes() + ad.make_nodes() + normal_dot_vel.make_nodes() + [flow_net.make_node(name='flow_network')] ) # Inlet BC (add T) inlet = PointwiseBoundaryConstraint( nodes=nodes, geometry=inlet_mesh, outvar={'u': u, 'v': v, 'w': w, 'T': T_inlet_nd}, batch_size=cfg.batch_size.inlet, ) # No-slip BC (add T) no_slip = PointwiseBoundaryConstraint( nodes=nodes, geometry=noslip_mesh, outvar={'u': 0, 'v': 0, 'w': 0, 'T': T_wall_nd}, batch_size=cfg.batch_size.no_slip, ) # Interior (add temperature residual) interior = PointwiseInteriorConstraint( nodes=nodes, geometry=interior_mesh, outvar={ 'continuity': 0, 'momentum_x': 0, 'momentum_y': 0, 'momentum_z': 0, 'advection_diffusion_T': 0, }, batch_size=cfg.batch_size.interior, ) # Post-process: recover physical temperature # T_physical = 100 + T_nd * (600 - 100) = 100 + T_nd * 500

# 4. Key Reminders

Verify residual key name: Print ad.make_nodes() outputs to confirm the key is 'advection_diffusion_T' in your PhysicsNeMo version.

inlet_normal: Must be verified from the actual STL geometry — the aneurysm value (0.8526, -0.428, 0.299) is likely wrong for your mesh.

outlet_area_raw: Measure from your STL file; the original value 12.0773 was for the NVIDIA example geometry.

Prandtl number: Pr=0.7 is used for simplicity (air-like). For actual blood flow revert to Pr=25. Lower Pr trains more easily in PINNs.

CCM reference run: Use T_inlet=600 degC and T_wall=100 degC as fixed temperature boundary conditions. These are consistent with the non-dimensional values in the PINN.